# 🌐 4–5: WEEKLY PRACTICE AND TROUBLESHOOTING

### 💻 Practical:

Resolve Connectivity Issues in a Complex Network

### 📝 Task:

Submit Troubleshooting Steps and Resolutions

# 🔹 TASK 1

### 📌 Exact Question:

Use the ping command to test connectivity between two PCs on different subnets in your Packet Tracer network and document the output for both successful and failed attempts.

### ✅ Answer:

I created two different subnets in Cisco Packet Tracer.

### 📊 Network Configuration:

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| 💻 PC1 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| 💻 PC2 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |

The router was configured to provide communication between the two subnets.

### 🟢 Successful Ping Test

From PC1, I tested connectivity to PC2:

ping 192.168.20.10

### Expected Output:

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

### 🔴 Failed Ping Test

I then simulated a connectivity problem by using an incorrect gateway on PC1.

For example, PC1 was configured with:

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.254

The ping test was:

ping 192.168.20.10

### Possible Output:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

### 🔧 Resolution:

I corrected the default gateway:

192.168.10.254  →  192.168.10.1

After correcting the gateway, I tested the ping again and connectivity was restored. ✅

### 💡 Conclusion:

A successful ping confirms communication between the two subnets. A failed ping can be caused by an incorrect IP address, subnet mask, default gateway, routing configuration, or cable/link problem.

# 🔹 TASK 2

### 📌 Exact Question:

Identify and fix an incorrect default gateway configuration on one device in your network so that it can access an external web server simulation; submit the steps you took and the command outputs before and after the fix.

### ✅ Answer:

I simulated a network where a PC had an incorrect default gateway.

### 📊 Initial Configuration:

| Setting | Incorrect Value |
| --- | --- |
| IP Address | 192.168.10.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.10.254 ❌ |
| Correct Gateway | 192.168.10.1 ✅ |

### 🔎 Step 1: Check IP Configuration

On the PC, I checked the IP configuration:

ipconfig

The default gateway was incorrect.

### 🔎 Step 2: Test the Gateway

ping 192.168.10.254

The test failed because this was not the correct router gateway.

### 🔎 Step 3: Test External Web Server

ping 203.0.113.10

The ping failed because the PC could not forward traffic through the correct gateway.

### 🔧 Step 4: Correct the Gateway

I changed the default gateway from:

192.168.10.254

to:

192.168.10.1

### 🟢 Step 5: Test Again

First, I tested the router:

ping 192.168.10.1

Result: Successful ✅

Then I tested the external web server:

ping 203.0.113.10

Result: Successful ✅

### 📊 Before vs After

| Test | Before Fix ❌ | After Fix ✅ |
| --- | --- | --- |
| Ping Gateway | Failed | Successful |
| Ping Web Server | Failed | Successful |
| Internet/External Access | Not Available | Available |

### 💡 Conclusion:

The problem was caused by an incorrect default gateway. Correcting the gateway allowed the PC to send traffic to the router and access the external web server.

# 🔹 TASK 3

### 📌 Exact Question:

A user reports they cannot access Flipkart.com from their simulated network. Trace the path using traceroute (or tracert) and identify where the connection fails; explain how you diagnosed and resolved the issue.

### ✅ Answer:

I used the tracert command to identify where the connection problem occurred.

### 💻 Step 1: Test Connectivity

ping www.flipkart.com

The connection was unsuccessful.

### 🔎 Step 2: Trace the Network Path

On Windows, I used:

tracert www.flipkart.com

The traceroute showed the path through different network hops.

### 🔍 Step 3: Identify the Failure

For example:

1    192.168.10.1       Reply

2    10.10.10.1         Reply

3    Request timed out.

4    Request timed out.

The connection stopped responding after the second hop.

### 🧠 Possible Causes:

- ❌ Incorrect routing

- ❌ Missing static/default route

- 🔥 ACL/firewall blocking traffic

- 🌐 ISP/network path problem

- ⚙️ Incorrect gateway or DNS configuration

### 🔧 Step 4: Check the Router

I checked the routing table:

show ip route

I found that the required route was missing.

### 🔧 Step 5: Correct the Route

I configured the required route/default route according to the topology.

Example:

Router(config)# ip route 0.0.0.0 0.0.0.0 10.10.10.2

### 🧪 Step 6: Verify

I checked the routing table again:

show ip route

Then tested:

ping www.flipkart.com

The connection was successful after correcting the route. ✅

### 💡 Conclusion:

Traceroute helped identify where communication stopped. After checking the routing table, I corrected the missing route and verified connectivity again.

Note: In a real network, the exact hop where tracert stops can vary. The example above demonstrates the troubleshooting method.

# 🔹 TASK 4

### 📌 Exact Question:

Simulate a scenario where a router has an incorrect static route that breaks connectivity between two VLANs. Correct the route configuration and verify the solution using appropriate show commands.

Hint: Use show ip route and show running-config to check current routes and settings.

### ✅ Answer:

I simulated a network with two VLANs connected through routers.

### 📊 Example Network:

VLAN 10                         VLAN 20

192.168.10.0/24               192.168.20.0/24

│                              │

Switch                         Switch

│                              │

Router R1  ────────────────  Router R2

### 🔴 Problem:

Router R1 had an incorrect static route to the VLAN 20 network.

### Step 1: Check the Routing Table

On R1:

show ip route

The route was pointing to the wrong next-hop address.

Example:

S 192.168.20.0/24 [1/0] via 10.10.10.5

But the correct next-hop address was:

10.10.10.2

### Step 2: Check Running Configuration

show running-config

I found the incorrect static route.

### 🔧 Step 3: Remove the Incorrect Route

Router(config)# no ip route 192.168.20.0 255.255.255.0 10.10.10.5

### 🔧 Step 4: Add the Correct Route

Router(config)# ip route 192.168.20.0 255.255.255.0 10.10.10.2

### Step 5: Verify the Route

show ip route

The correct route was now displayed:

S 192.168.20.0/24 [1/0] via 10.10.10.2

### Step 6: Test Connectivity

From a PC in VLAN 10:

ping 192.168.20.10

The ping was successful. ✅

### 📊 Before vs After

| Check | Before Fix ❌ | After Fix ✅ |
| --- | --- | --- |
| Static Route | Incorrect | Correct |
| show ip route | Wrong next hop | Correct next hop |
| VLAN-to-VLAN Ping | Failed | Successful |
| Connectivity | Broken | Restored |

### 💡 Conclusion:

The problem was caused by an incorrect static route. After replacing it with the correct route, communication between the two VLANs was restored.

# 🔹 TASK 5

### 📌 Exact Question:

You receive a network diagram and a set of running-config outputs for a multi-router setup where WhatsApp voice calls are not working between two branches. Analyze the configs, identify the misconfiguration, and describe your troubleshooting steps and resolution.

### ✅ Answer:

I would troubleshoot the problem systematically because voice communication requires proper IP connectivity, routing, and required traffic to be allowed.

### 🗺️ Example Network:

🏢 Branch A

192.168.10.0/24

│

Router R1

│

WAN Link

│

Router R2

│

🏢 Branch B

192.168.20.0/24

### 🔎 Step 1: Check IP Connectivity

From Branch A, I would ping the Branch B router/interface:

ping 192.168.20.1

If the ping fails, I would troubleshoot the network before checking the application.

### 🔎 Step 2: Check Routing Tables

On both routers:

show ip route

I would check whether each router has a route to the other branch's network.

For example:

R1 should know:

192.168.20.0/24

R2 should know:

192.168.10.0/24

### 🔎 Step 3: Check the Running Configuration

I would use:

show running-config

I would check:

- IP addresses

- Default gateways

- Static routes

- ACLs

- NAT configuration

- Interface status

### 🔥 Step 4: Check ACL Configuration

If routing is correct but voice traffic is still not working, I would check whether an ACL is blocking required traffic.

show access-lists

A wrongly configured ACL could allow normal web traffic but block some required communication.

### 🔧 Step 5: Identify and Correct the Misconfiguration

### Example Problem:

R1 had an incorrect static route:

ip route 192.168.20.0 255.255.255.0 10.10.10.5

But the correct next-hop router was:

10.10.10.2

### Correct Configuration:

R1(config)# no ip route 192.168.20.0 255.255.255.0 10.10.10.5

R1(config)# ip route 192.168.20.0 255.255.255.0 10.10.10.2

### 🧪 Step 6: Verify the Fix

Check the routing table:

show ip route

Then test the remote branch:

ping 192.168.20.1

If connectivity is successful, the routing problem has been resolved.

### 📊 Troubleshooting Summary

| Step | Command/Action | Purpose |
| --- | --- | --- |
| 1️⃣ | ping | Check basic connectivity |
| 2️⃣ | show ip route | Check routing |
| 3️⃣ | show running-config | Check router configuration |
| 4️⃣ | show access-lists | Check traffic filtering |
| 5️⃣ | Correct route/ACL | Fix the misconfiguration |
| 6️⃣ | ping again | Verify connectivity |

### 💡 Conclusion:

The voice-call problem was diagnosed by checking connectivity, routing tables, running configurations, and ACLs. In the simulated example, the problem was caused by an incorrect static route. Correcting the route restored communication between the two branches.

Important: In a real WhatsApp voice-call issue, successful ping does not guarantee that voice calls will work. Application-specific requirements, NAT, firewall/ACL behavior, and the actual network path would also need to be checked.

# 🛠️ IMPORTANT CISCO TROUBLESHOOTING COMMANDS

| Command | Purpose |
| --- | --- |
| ping <IP> | 📡 Test connectivity |
| tracert <domain> | 🗺️ Trace path on Windows |
| show ip route | 🛣️ Check routing table |
| show running-config | ⚙️ Check current configuration |
| show ip interface brief | 🔌 Check interface status and IPs |
| show access-lists | 🔥 Check ACL rules |
| show arp | 🔎 Check IP-to-MAC mappings |
| show cdp neighbors | 🔗 Check directly connected Cisco devices |

# 🎯 OVERALL TROUBLESHOOTING METHOD

When troubleshooting a complex network, I would follow this order:

🔌 1. Check Physical Connections

↓

💻 2. Check IP Address & Subnet Mask

↓

🚪 3. Check Default Gateway

↓

📡 4. Test Using Ping

↓

🗺️ 5. Check Routing / Traceroute

↓

⚙️ 6. Check Router Configuration

↓

🔥 7. Check ACL / Firewall

↓

🔧 8. Fix the Problem

↓

🧪 9. Test Connectivity Again

↓

✅ 10. Confirm the Resolution

# 🏆 FINAL CONCLUSION

Network troubleshooting should always be performed step-by-step and logically. Commands such as ping, tracert, show ip route, and show running-config help identify whether a problem is related to the device, IP configuration, gateway, routing, ACL, or network path.

The most important troubleshooting principle is:

“Identify the problem → Find the cause → Apply the correct fix → Verify the result.”

This approach helps network administrators resolve connectivity problems efficiently and maintain reliable communication between different networks and branches.